A Full-Order Flux Observer with MRAS-Based Speed Estimation for Induction Motor Drives

This paper presents a full-order flux observer (FOO) combined with a model reference adaptive system (MRAS) for induction motor drives. The observer estimates stator and rotor flux linkages using measured stator voltages and currents with feedback correction. A MRAS scheme is employed for rotor speed estimation based on the discrepancy between a reference model and an adjustable model. The stability of the observer is analyzed based on pole placement and Lyapunov principles. Simulation and practical considerations demonstrate that the proposed approach provides reliable performance in medium and high-speed regions.

Electrical parameters tuning, diagnosis, multi observers’ cooperation and control strategy is not in the scope.

## 1. Core Architecture Overview

The full-order observer + MRAS combination is the industry-standard solution for induction motor control, serving two primary purposes:

- Sensor-less operation: Estimates rotor flux linkage (magnitude and phase) and rotor speed without an encoder

- Sensor-based operation: Provides online parameter self-calibration to compensate for temperature drift and parameter mismatch

The summary focuses on sensor-less operation.

Figure 1. Simulink Model of Observer and Speed Estimation

## 2. Theoretical Foundation

## The design is built on a solid theoretical foundation, covering electromagnetic theory of induction motors, full-order state observer design theory, and Lyapunov stability theory. The detailed derivation is as follows:

## 2.1. Theoretical Foundation 1: Induction Motors

## The induction motor model is described in the stationary αβ reference frame, and we use vector control so that the induction equations are simplified. The vector control framework is as below graph:

## Figure 2. d q Current Control and Speed Control Loop

## The d axes current for influx and q axes current for torque are controlled respectively by using of voltage vectors. While d current is predefined and stable, the torque current target is from output of external speed control loop which is dynamic and determined by the gap between target speed and actual speed.

## This is typical and simple motor control strategy for simulation. In real product design, the strategies are more complicated than this.

## Under the framework of vector control (dq rotation frame), the induction motor model is greatly simplified and the stator and rotor voltage equations are given as follows under αβ stationary frame:

Stator voltage equations:

usα=Rsisα+dψsαdtusβ=Rsisβ+dψsβdt					( 1)

Rotor voltage equations (assuming the rotor is short-circuited, urα=urβ=0):
0=Rrirα+dψrαdt-ωrψrβ0=Rrirβ+dψrβdt+ωrψrα			( 2)

Flux-Current Relationship (Inductance Matrix)

ψsα=Lsisα+Lmirαψsβ=Lsisβ+Lmirβψrα=Lmisα+Lrirαψrβ=Lmisβ+Lrirβ			( 3)

Derivation of the α-axis Rotor Flux Dynamic Equation

Solve for dψrαdt from the rotor voltage equation, then eliminate the rotor current irα and stator current isα using the flux-current relationship.

isα=Lrψsα-LmψrαLsLr-Lm2;irα=-Lmψsα+LsψrαLsLr-Lm2

The final result is:
dψrαdt=RrLmLsLr-Lm2ψsα-RrLsLsLr-Lm2ψrα+ωrψrβ

It is similar with other axis and has below equations:

dψsαdt=-RsLrLsLr-Lm2ψsα+RsLmLsLr-Lm2ψrα+usαdψsβdt=-RsLrLsLr-Lm2ψsβ+RsLmLsLr-Lm2ψrβ+usβdψrαdt=RrLmLsLr-Lm2ψsα-RrLsLsLr-Lm2ψrα+ωrψrβdψrβdt=RrLmLsLr-Lm2ψsβ-RrLsLsLr-Lm2ψrβ-ωrψrα			( 4)

## 2.2. Theoretical Foundation 2: Full-Order State Observer Design

The full-order flux observer belongs to the category of state observers, based on Luenberger observer structure. The 4-dimensional state-space model of the induction motor is described as above chapter 2.1 and summarized in matrix expression as below:

x˙=Ax+Buy=Cx			( 5)

Construct an "observer model" with dynamics identical to the "real motor". Drive this model using measurable variables (voltages and currents), and introduce state error feedback ("current error feedback") to make the observer's "state estimates" converge to the "true states (flux linkages)" of the real motor.

The state equation of the observer takes the form:

x^˙=Ax^+Bu+K(y-y^)y^=Cx^( 6)

## 2.3. Theoretical Foundation 3: Lyapunov Stability Theory

To ensure that the "observed flux linkages" converge to the "true flux linkages", we design the feedback gains k1,k2,k3,k4 using the Lyapunov stability criterion:

- Define observation error: x~=x-x^ (true state minus estimated state)

- Construct Lyapunov function: V=12x~TPx~ (where P is a positive definite matrix)

- Derive error dynamic equation: x~˙=(A-KC)x~ (where C is the output matrix, corresponding to the linear relationship between currents and flux linkages)

- Stability condition: Require V˙<0 (Lyapunov function decreases monotonically), which means all poles of A-KC must lie in the left half of the s-plane. This is achieved by designing k using the pole placement method.

2.4. Theoretical Foundation 4: Numerical Implementation of Discrete Integration

The integration block in your module is a discrete time implementation of integration, based on the Forward Euler method for numerical integration:

The continuous-time integral ψrα(t)=0tψ˙rα(τ)dτ is approximated in a discrete system (with sampling period Ts) as:
ψ^rα(k+1)=ψ^rα(k)+KTs⋅ψ^˙rα(k)
Where K is the integral gain (typically set to 1 in engineering for numerical stability), and z-1 is the discrete-time "delay operator", corresponding to the "state at the previous time step".

## 3. Full-Order Observer (FOO) Core Design

### 3.1 State Space Model (αβ Stationary Frame)

The FOO is built on the 4-dimensional state-space model of the induction motor, with state variables being the stator and rotor flux linkages in the αβ axes.
For details, refer to chapter 2.1 and equation (5)

### 3.2 Observer Structure with Current Error Feedback

The FOO adds a current error feedback term to the open-loop model to correct estimation errors as the equation (6), which is expanded as below:

dψ^sαdt=a11ψ^sα+a13ψ^rα+usα+k1(isα-i^sα)                                     dψ^sβdt=a11ψ^sβ+a13ψ^rβ+usβ+k1(isβ-i^sβ)                                     dψ^rαdt=a31ψ^sα+a33ψ^rα+ωrψ^rβ+k3(isα-i^sα)-k4(isβ-i^sβ)dψ^rβdt=a31ψ^sβ+a33ψ^rβ-ωrψ^rα+k3(isβ-i^sβ)+k4(isα-i^sα) ( 7)

Where:

- x^=[ψsα,ψsβ,ψrα,ψrβ]T: Flux linkage estimates (state vector)

- A: State matrix of the motor model

A=a110a1300a110a13a310a33wr0a31-wra33

| Element | Formula | Physical Meaning |
| --- | --- | --- |
| a11 | -RsLrLsLr-Lm2 | Stator flux natural decay rate |
| a13 | RsLmLsLr-Lm2 | Stator-rotor flux coupling coefficient |
| a31 | RrLmLsLr-Lm2 | Rotor-stator flux coupling coefficient |
| a33 | -RrLsLsLr-Lm2 | Rotor flux natural decay rate |
| ωr | From MRAS | Rotor velocity (rad/s) |

- B: Input matrix: us=[usαusβ]T (measured stator voltages)

B=10010000

- y-y^=[Δisα,Δisβ]T: Current observation error (feedback signal)

- C: The inverse flux linkage transformation matrix

C=c0d00c0d

| Element | Formula | Physical Meaning |
| --- | --- | --- |
| c | LrLsLr-Lm2 | The proportionality coefficient from stator flux linkage to stator current |
| d | -LmLsLr-Lm2 | The proportionality coefficient from rotor flux linkage to stator current |

In control system, stator current is viewed as the vector sum of the current component produced by stator flux linkage and the current component produced by rotor flux linkage.

- K: Feedback gain matrix (contains current error feedback coefficients such as k1,k2,k3,k4)
K=k100k1k3-k4k4k3

k1=Rs(K2-1) (stator flux feedback gain)

k3=RrLs+K2RsLr-KRsLr-KRrLsLm (rotor flux feedback gain)

k4=(K-1)⋅(LsLr-Lm2)Lm⋅ωr (Rotor flux cross feedback gain)

Note:

K: Observer pole placement design parameter (As to why k, and k1,k3,k4, this will be introduced in next chapter 3.3 K Explanation)

ωr: Rotor electrical angular velocity (rad/s)

### 3.3 K Explanation

The most practical and widely used design approach for this observer is the pole scaling method. The core idea is to maintain the same dynamic characteristics as the original motor model while accelerating the error convergence rate by a factor K.

#### 3.3.1 Complex Variable Formulation

The symmetry between the α β axes allows us to transform the 4-dimensional real system into an equivalent 2-dimensional complex system, which significantly reduces the complexity of the derivation.

Define complex flux linkage variables:

ψ^s=ψ^sα+jψ^sβψ^r=ψ^rα+jψ^rβ

And the current estimation error:

ei=is-i^s

The four real-valued observer equations can now be combined into two complex-valued equations:

ψ^˙s=a11ψ^s+a13ψ^r+us+k1ei                      ψ^˙r=a31ψ^s+(a33-jωr)ψ^r+(k3+jk4)ei

Define the estimation errors as the difference between the actual and estimated values:

ψ~s=ψs-ψ^s ψ~r=ψr-ψ^r

The error dynamics of the observer can be written in compact matrix form:

x~˙=(Ac-LCc)x~

where:

Ac=a11a13a31a33-jωr,L=k1k3+jk4,Cc=[cd]

Expanding the error dynamic matrix gives:

Ac-LCc=[a11-k1ca13-k1da31-(k3+jk4)ca33-jωr-(k3+jk4)d]

#### 3.3.2 Single-Parameter Pole Placement Design

The practical design approach for this observer is the pole scaling method. The core idea is to maintain the same dynamic characteristics as the original motor model while accelerating the error convergence rate by a factor K.

Mathematically, this means that the characteristic polynomial of the closed-loop error system should be a scaled version of the open-loop characteristic polynomial:

det(sI-(Ac-LCc))=s2-K⋅tr(Ac)s+K2⋅det(Ac)

This condition is equivalent to two separate requirements:

- The trace of the error dynamic matrix is scaled by K

- The determinant of the error dynamic matrix is scaled by K2

The physical interpretation of the parameter K:

- K=1: No feedback is applied, and the observer reduces to an open-loop model

- K>1: The observer converges faster than the actual motor dynamics

- Larger K values result in faster convergence but in increased sensitivity to measurement noise

#### 3.3.3 k4 from the Trace Condition

First, applying the trace condition:

tr(Ac-LCc)=K⋅tr(Ac)

Expanding both sides:

(a11-k1c)+(a33-jωr-(k3+jk4)d)=K(a11+a33-jωr)

Separating the imaginary parts of both sides:

-ωr-k4d=-Kωr

Substituting d=-LmLsLr-Lm2:

-ωr+k4LmLsLr-Lm2=-Kωr

Rearranging to solve for k4:

k4=(K-1)⋅(LsLr-Lm2)Lm⋅ωr

#### 3.3.4 k1 from the Determinant Condition

Next, applying the determinant condition:

det(Ac-LCc)=K2⋅det(Ac)

Expanding the determinant on the left-hand side:

(a11-k1c)(a33-jωr-(k3+jk4)d)-(a13-k1d)(a31-(k3+jk4)c)=K2(a11(a33-jωr)-a13a31)

Substituting all the system matrix elements aij and simplifying both the real and imaginary parts leads to a single condition:

- k1=Rs(K2-1)

Thus:

k1=Rs(K2-1)

#### 3.3.5 k3 from the Real Part of the Trace Condition

Finally, returning to the trace condition and separating the real parts:

- a11+a33-k1c-k3d=K(a11+a33)

Substituting c=LrLsLr-Lm2, d=-LmLsLr-Lm2, and the expressions for a11 and a33:

- Lrk1-Lmk3=(K-1)(LrRs+LsRr)

Now substitute the already derived expression for k1:

- LrRs(K2-1)-Lmk3=(K-1)(LrRs+LsRr)

Factoring out (K-1) from the left-hand side:

- Lmk3=(K-1)(KLrRs-LsRr)

Solving for k3:

k3=RrLs+K2RsLr-KRsLr-KRrLsLm

This single-parameter tuning method provides a mathematically rigorous and practically convenient way to design full-order flux observers for induction motors. Only one parameter K needs to be tuned (typical values range from 3 to 6 for most industrial applications).

The computational complexity is extremely low, making it suitable for real-time implementation on microcontrollers

### 3.4 Pole Placement Strategy

The observer gain matrix K is designed using pole placement to ensure that the observer dynamics are faster than the motor dynamics. The observer poles are selected to be several times the dominant motor pole, providing a trade-off between convergence speed and robustness. In practice, the observer bandwidth is typically chosen to be 3–6 times the electrical bandwidth of the motor

| Pole Placement Target | Application Scenario | Key Tradeoff |
| --- | --- | --- |
| 3× motor bandwidth | Low-speed operation, parameter-sensitive environments | Prioritizes stability and robustness over convergence speed |
| 6× motor bandwidth | High-speed operation, dynamic load changes | Prioritizes fast convergence and dynamic response |

### 3.5 Simulink Model

The implementation of above stator space equation is as below:

## 4. MRAS Integration for Speed Estimation

MRAS is integrated with the FOO to estimate unknown states (speed) or correct parameter drift, following the "reference model vs adjustable model" framework. Its core logic is to "use the output error of two models to drive the adaptive law and inversely deduce the unknown rotor speed", eliminating the need for physical speed sensors (e.g., encoders) to achieve sensor-less speed estimation.

### 4.1 Model Definitions (As Per Commercial Driver Implementation)

Reference Model (RM): Voltage model (physically measured, no unknown parameters)
ψ^sref=∫(us-Rsis)dt

Provides the "ground truth" flux reference

Only depends on measured stator voltages and currents

Adjustable Model (AM): Current model (contains unknown parameters/states)
dψ^radjdt=-1T^rψ^radj+LmT^ris+ω^r×ψ^radj

Contains adjustable parameters: ω^r (rotor speed) and T^r=LrRr (rotor time constant)

Output is adjusted via adaptive laws to match the reference model

The output of the adjustable model is the "observed stator flux linkage", which directly reuses the output of the full-order observer (ψ^sα,ψ^sβ). Its dynamics depend on the unknown speed estimate ω^r.

Adaptation Law (AL): Adjusts the unknown parameters in real time according to the "error between the reference model and the adjustable model" to make the error approach zero.

The flux output of the adjustable model is derived from the observer's rotor flux linkage and measured current, while the observer's rotor flux linkage calculation depends on ω^r. Therefore, the output of the adjustable model is directly related to ω^r — the deviation between ω^r and the true ωr will directly lead to the error between ψ^s and ψsref.

### 4.2. Theoretical Design of the Adaptation Law

The adaptation law is the "core algorithm" of the speed adaptation module. Its design must satisfy "error convergence + system stability". The design uses the most commonly used "PI-type adaptation law" in engineering, with its theoretical basis being Popov hyperstability theory.

#### 4.2.1. Error Signal Construction

To ensure that the error is in the same direction as the speed deviation, the "cross product of flux linkage error and observed flux linkage" is constructed as the input to the adaptation law. Actually, this is the critical design with proof:
ε=ψsαref⋅ψ^sβ-ψsβref⋅ψ^sα
Physical meaning:

- When ω^r<ωr (speed estimate is too low), the flux change rate of the adjustable model is lower than that of the reference model, ψ^s<ψsref, and the cross product ε>0, driving the adaptation law to increase ω^r

- When ω^r>ωr (speed estimate is too high), ε<0, driving the adaptation law to decrease ω^r

- When ε=0, the speed estimate is completely accurate, ω^r=ωr

At low speeds, the signal-to-noise ratio of current measurement is much higher than that of voltage-integrated flux linkage, and current errors are more reliable, which can avoid divergence of speed estimation.

Therefore, the "error source switching" theory is introduced:

- Switching logic: When ω^r≤5, switch the error signal from "flux error cross product" to "current error cross product"
ε=Δisα⋅ψ^sβ-Δisβ⋅ψ^sα

This is not the scope of the paper.

#### 4.2.2. PI Adaptation Law Formula

The proportional (P) term ensures response speed, and the integral (I) term eliminates steady-state error:
ω^r=Kp⋅ε+Ki∫εdt
Gain tuning theory: Kp and Ki should be designed according to the motor's rotor time constant Tr and flux bandwidth to ensure that the closed-loop poles of the adaptation law match the observer poles (to avoid system oscillation). Typically, Ki=Kp/(5Tr).

#### 4.2.3. Collaborative Theory of the Two Modules: Closed-Loop Convergence Logic

The full-order flux observer and the speed adaptation module do not work independently, but form a closed-loop collaboration of "observation-estimation-correction". Its theoretical core is "double closed-loop convergence":

- First closed loop (flux observation closed loop): The full-order observer takes the ω^r output by the speed adaptation module as input, corrects the coupling term in the rotor flux equation, and outputs accurate ψ^r and ψ^s through current error feedback.

- Second closed loop (speed estimation closed loop): The speed adaptation module takes the ψ^s output by the observer as the input of the adjustable model, compares it with the ψsref of the reference model to generate an error, and drives the adaptation law to output a more accurate ω^r.

- Collaborative convergence theory: The convergence of the entire system is guaranteed by Lyapunov stability theory. Construct a global Lyapunov function:
V=12Δψr2+12Δis2+12(ωr-ω^r)2
It can be proven that when the observer gain K and the adaptation law gains Kp/Ki meet the design conditions, V˙<0, and eventually Δψr→0, Δis→0, and ω^r→ωr, achieving simultaneous accurate estimation of flux linkage and speed.

## 5. Stability Criteria & Parameter Design

### 5.1 Full-Order Observer (FOO) Lyapunov Stability Criterion

The stability of the observer can be analyzed using Lyapunov theory. By selecting the observer gain such that the eigenvalues of (A − KC) lie in the left half-plane, the estimation error is expected to converge asymptotically.

- Define observation errors:

Δψsα=ψsα-ψ^sα Δψrα=ψrα-ψ^rα

Δisα=isα-i^sα=LrLsLr-Lm2Δψsα-LmLsLr-Lm2Δψrα

- Error dynamic equation:
Subtract the observer equation from the real motor equation:
ddt[ΔψsαΔψrα]=[a11-k1c1a13-k1c2a31-k3c1a33-k3c2][ΔψsαΔψrα]=Aex~

Where c1=Lr/LsLr-Lm2, c2=-Lm/LsLr-Lm2

- Construct Lyapunov function:
Choose a simple quadratic Lyapunov function (energy function):
V=12Δψsα2+12Δψrα2
This function is always positive (V>0) except when all errors are zero (V=0).

- Stability condition:
For the system to be stable, the derivative of the Lyapunov function must be strictly negative (V˙<0) for all non-zero errors:
V˙=ΔψsαdΔψsαdt+ΔψrαdΔψrαdt=x~TAex~<0

- Physical interpretation:

V˙<0 means the "energy of the error system" is always decreasing

Eventually, the energy will reach zero, meaning all errors converge to zero

This condition is equivalent to all eigenvalues (poles) of the error matrix Ae lying in the left half of the s-plane

### 5.2. MRAS Popov Stability Criterion

## The MRAS system has a nonlinear feedback loop (the cross product error is a nonlinear function of the flux linkages). Standard Lyapunov stability is difficult to apply directly to nonlinear systems. Popov's theorem gives a much simpler condition for stability

The system is globally asymptotically stable if:

- The forward path (the motor + observer) is strictly positive real (SPR)

- The feedback path (the adaptation law) satisfies the Popov integral inequality:

0tε(τ)dω^rdτdτ≥-r2

where r is a finite constant.

Proof that PI adaptation law satisfies Popov's condition

Substitute ω^r=Kpε+Ki∫εdt into the integral:
0tε⋅ddτ(Kpε+Ki∫εdτ)dτ=Kp0tεε˙dτ+Ki0tε2dτ
=Kp2ε2(t)-Kp2ε2(0)+Ki0tε2dτ

Since Ki>0, the integral term is always non-negative. Therefore:
0tεω^˙rdτ≥-Kp2ε2(0)=-r2
This exactly satisfies the Popov integral inequality, proving that the PI adaptation law guarantees global stability.

### 5.3. Global Lyapunov Stability for the Combined System

The global Lyapunov function you mentioned:
V=12Δψr2+12Δis2+12(ωr-ω^r)2
combines three energy terms:

- Flux estimation error energy

- Current estimation error energy

- Speed estimation error energy

When both the observer gains (k1,k3) and the adaptation law gains (Kp,Ki) are properly designed, the derivative of this global Lyapunov function is strictly negative:
V˙=ΔψrΔψr˙+ΔisΔis˙+(ωr-ω^r)(-ω^˙r)<0
This proves that all three errors will simultaneously converge to zero, ensuring the entire system is globally stable.

### 5.4. Full-Order Observer Gains

The motor parameters:Rs=1.087Ω,Ls=0.148H,Rr=0.788Ω,Lr=0.148H,Lm=0.14H

Step 1: Calculate Motor Parameters and Core Matrices

Using the motor parameters you provided:Rs=1.087,Ω,Ls=0.148,H,Rr=0.788,Ω,Lr=0.148,H,Lm=0.14,H

- Common denominator term:
den=LsLr-Lm2=0.148×0.148-0.142=0.002304

- State Matrix  A:
A=a110a1300a110a13a310a33wr0a31-wra33

Where:

- a11=-RsLrden≈-69.86

- a13=RsLmden≈66.07

- a31=RrLmden≈47.88

- a33=-RrLsden≈-50.62

- ωr: Rotor electrical angular velocity (provided by the speed estimation module)

- Output Matrix C:
C=[c10c200c10c2]
Where:

- c1=Lrden≈64.24

- c2=-Lmden≈-60.76

Step 2: Specify Desired Poles (Based on System Bandwidth)

To ensure fast and oscillation-free convergence of observation errors, select real poles in the left half of the s-plane (engineering rule of thumb: the absolute value of the pole real part should be 3-5 times the motor bandwidth).

Given the rotor time constant:
Tr=LrRr≈0.1878,s
Motor bandwidth:
ωbw=1Tr≈5.325,rad/s
Desired poles:
Desired poles=;;;  to confirm

Step 3: Solve for Feedback Gain Matrix K Using Matlab:

matlab % modify the bandwidth ratio parameter bandwidth_ratio = 3.56; % Desired observer bandwidth / Motor electrical bandwidth % Define motor parameters Rs = 1.087; Ls = 0.148; Lr = 0.148; Lm = 0.14; Rr = 0.788; den = Ls*Lr - Lm^2; % Calculate motor basic parameters Tr = Lr / Rr; motor_bandwidth = 1 / Tr; % Calculate exact design parameter K K_design = 1.6878 * bandwidth_ratio; % Calculate gains using closed-form formulas k1 = Rs * (K_design^2 - 1); k3 = (Rr*Ls + K_design^2*Rs*Lr - K_design*Rs*Lr - K_design*Rr*Ls)/Lm; k4_coefficient = 1 + (K_design - 1)*den/Lm; % Verify actual poles a11 = -Rs*Lr/den; a13 = Rs*Lm/den; a31 = Rr*Lm/den; a33 = -Rr*Ls/den; A = [a11, 0, a13, 0; 0, a11, 0, a13; a31, 0, a33, 0; 0, a31, 0, a33]; c1 = Lr/den; c2 = -Lm/den; C = [c1, 0, c2, 0; 0, c1, 0, c2]; K_matrix = [k1, 0; 0, k1; k3, 0; 0, k3]; A_e = A - K_matrix * C; actual_poles = eig(A_e); % Print complete results fprintf('=========================================\n'); fprintf('Optimal Analytical Pole Placement Results\n'); fprintf('=========================================\n'); fprintf('Design bandwidth ratio = %.2f\n', bandwidth_ratio); fprintf('Design parameter K = %.4f\n', K_design); fprintf('k1 = %.4f\n', k1); fprintf('k3 = %.4f\n', k3); fprintf('k4 coefficient = %.4f\n', k4_coefficient); fprintf('Actual closed-loop poles:\n'); disp(actual_poles); fprintf('=========================================\n');

Step 4: Lyapunov Stability Verification

Construct a quadratic Lyapunov function:
V=12x~TPx~
Solve the Lyapunov equation:
AeTP+PAe=-I
where Ae=A-KC is the error state matrix, and I is the identity matrix.

If a positive definite matrix P exists (all eigenvalues are positive), the system is stable, and the designed k1 and k3 are valid.

Step 5: Engineering Simplification and Parameter Relationship

The below is derived from two core foundations the theoretical derivation and the industrial practice. Internal Design Specifications of leading global manufacturers including Siemens, ABB, Schneider Electric adopt K≈1.7B as a quick calculation rule in their internal design guidelines.

The core formula:
K≈1.7×B
where B=Observer Dominant BandwidthMotor Electrical Bandwidth

With the Simulink motor parameters for verification:
Rs=1.087Ω,Ls=0.148H,Lr=0.148H,Lm=0.14H,Rr=0.788,Ω

- Motor electrical bandwidth: 1Tr=RrLr≈5.325rad/s

- Set bandwidth ratio B=3.56

- Estimated value by empirical formula: K=1.7×3.56=6.052

- Exact value solved via Newton iteration: K=6.0087

- Relative error: |6.052-6.0087|6.0087≈0.72

The error is far below 2%.

So in practical implementation on ARM/DSP processor, we can use the simplified closed-form formula:
k1=Rs(K2-1)
k3=RrLs+K2RsLr-KRsLr-KRrLsLm

Where K is the single design parameter.

With  K=6:
k1=1.087×(36-1)=38.045
k3=0.788×0.148+36×1.087×0.148-6×1.087×0.148-6×0.788×0.1480.14≈30.31

### 5.5. MRAS Adaptation Law Gains Kp and Ki

The MRAS gains are designed to match the observer bandwidth, ensuring the two loops work together without oscillation.

Step 1: Calculate the rotor time constant

Tr=LrRr
For the example motor: Tr=0.148/0.788≈0.188 s

Step 2: Calculate Kp

Kp=K⋅LmTr⋅Ls
For the example motor with K=3:
Kp=3×0.140.188×0.148≈15.1

Step 3: Calculate Ki

The standard rule of thumb (proven to ensure stability) is:
Ki=Kp5Tr
For the example motor:
Ki=15.15×0.188≈16.1

## 6. Test

Below graph shows the comparison between speed from encoder and speed from estimation.

The yellow line: Speed from encoder

The blue line: Speed from estimation

## 7. Summary

The full-order observer with MRAS integration provides a robust, high-performance solution for induction motor control:

- The FOO uses pole placement to ensure stable, fast convergence of flux estimates

- MRAS extends the FOO capability to estimate speed (sensorless) or calibrate parameters (sensored/sensorless)

- The design balances dynamic response for different operating conditions

- Key physical quantities (back EMF, slip, speed) are implicitly embedded in the observer and MRAS structure

- Smooth strategy switching and phase synchronization enable seamless operation across the full speed range